# Road Pothole Detection & Severity Analysis

Main Code - for project team review

Generated: 23 Sep 2026

Live demo: https://pothole-detection-api-h1cp.onrender.com/dashboard

Repository: https://github.com/MohdJasimAli/pothole-detection

# Overview

The system detects potholes in road images and estimates their size, depth
and severity to support smart road maintenance.

Pipeline (5 steps, implemented in main.py -> PotholeAnalysisPipeline):
  1. DETECT  - YOLOv8 object detection (or the classical CV fallback when
               PyTorch is unavailable). Output: bounding boxes + confidence.
  2. SIZE    - Convert pixel box to real-world cm using a calibrated baseline
               or a reference object; classify Small / Medium / Large.
  3. DEPTH   - MiDaS monocular depth map sampled inside each box; classify
               Shallow (<2cm) / Moderate (2-5cm) / Deep (>5cm).
  4. SEVERITY- Weighted score 0-100 from size + depth -> Critical / High /
               Medium / Low with a maintenance response deadline.
  5. REPORT  - Annotated image (severity-coloured boxes) + JSON report with
               per-pothole metrics, summary counts and backend info.

Severity -> response time:
  Critical (>=80): 24 hours | High (60-79): 1 week
  Medium (35-59): 1 month  | Low (<35): 3 months

Verified performance:
  - 24 tests passed, 1 skipped (integration test requires torch)
  - POST /api/analyze on a 3-pothole image: HTTP 200 in ~0.85s local,
    ~3.7s on the free cloud instance; all 3 potholes detected with
    independent size/depth/severity.

# How to Run

Run locally:
    pip install -r requirements.txt
    pytest tests/ -q
    python main.py --image data/samples/multi_potholes.jpg
    python main.py --web          # then open http://localhost:5000/dashboard

Deploy (already done):
    Live dashboard: https://pothole-detection-api-h1cp.onrender.com/dashboard
    Render auto-deploys on every push to main (build runs the test suite).

Train a custom YOLOv8 model:
    pip install torch ultralytics
    python train.py --create-config
    python train.py --data dataset.yaml --epochs 100 --model yolov8s.pt

# File Index

config.py - Configuration: paths, thresholds, size/depth/severity levels, API vars

main.py - MAIN PIPELINE - detect -> size -> depth -> severity -> report

src/detector.py - YOLOv8 detector + DetectionResult data structure (primary backend)

src/classical_detector.py - Classical CV detector: Otsu+adaptive threshold, morphology, contours

src/preprocessing.py - Image preprocessing: CLAHE contrast, denoise, lighting/shadow removal

src/depth_estimator.py - Depth estimation: MiDaS DPT + synthetic fallback, depth categories

src/size_estimator.py - Pixel -> cm size estimation (camera model) + Small/Medium/Large

src/severity.py - Severity scoring (0-100) + matrix classification + priority levels

src/utils.py - Helpers: logging, image IO, drawing boxes, IoU, report generation

api/app.py - Flask REST API: /api/analyze, /api/detect, /api/reports, /dashboard

train.py - YOLOv8 training script (dataset config, train, validate, export)

tests/test_detector.py - Unit tests - 25 tests covering config, preprocessing, size, severity

# config.py

Configuration: paths, thresholds, size/depth/severity levels, API vars

"""

Configuration module for Road Pothole Detection and Severity Analysis System.

Contains all configuration constants, paths, and settings.

"""

import os

# ============================================================

# Device Detection

# ============================================================

def _torch_usable():

"""Return True only if torch is installed AND its native libs load.

The probe runs in a subprocess because a broken torch install can raise

beyond ImportError/OSError (segfault / access violation), which would

otherwise crash this interpreter during a plain `import torch`.

"""

try:

from importlib import metadata as _metadata

_metadata.version("torch")

except Exception:

return False

try:

import subprocess as _sp

import sys as _sys

r = _sp.run(

[_sys.executable, "-c",

"import torch; "

"assert hasattr(torch, 'cuda'); "

"torch.cuda.is_available()"],

capture_output=True, timeout=120,

)

return r.returncode == 0

except Exception:

return False

try:

_TORCH_OK = _torch_usable()

except Exception:

_TORCH_OK = False

if _TORCH_OK:

import torch

DEFAULT_DEVICE = "cuda" if torch.cuda.is_available() else "cpu"

TORCH_AVAILABLE = True

else:

TORCH_AVAILABLE = False

DEFAULT_DEVICE = "cpu"

# ============================================================

# Project Paths

# ============================================================

PROJECT_ROOT = os.path.dirname(os.path.abspath(__file__))

# Data directories

DATA_DIR = os.path.join(PROJECT_ROOT, "data")

MODELS_DIR = os.path.join(DATA_DIR, "models")

SAMPLES_DIR = os.path.join(DATA_DIR, "samples")

UPLOADS_DIR = os.path.join(PROJECT_ROOT, "frontend", "uploads")

# Model paths

YOLO_MODEL_PATH = os.path.join(MODELS_DIR, "best.pt")

MIDAS_MODEL_PATH = os.path.join(MODELS_DIR, "midas")

# Detection backend mode. When True (default), the pipeline falls back to the

# classical image-processing detector (src/classical_detector.py) on machines

# where PyTorch/Ultralytics cannot be installed - for example sandboxed VMs

# where the torch native DLLs cannot initialize. When False, missing torch

# raises ImportError instead.

ALLOW_CLASSICAL_FALLBACK = True

# ============================================================

# Detection Settings

# ============================================================

# YOLO detection thresholds

CONFIDENCE_THRESHOLD = 0.4      # Minimum confidence for detection

IOU_THRESHOLD = 0.45            # NMS threshold

IMAGE_SIZE = 640                # Input image size for YOLO

# Classes to detect (0 = pothole in our trained model)

CLASS_NAMES = {0: "Pothole"}

# ============================================================

# Image Processing Settings

# ============================================================

# Camera calibration parameters (for size estimation)

# These should be calibrated based on your camera setup

CAMERA_HEIGHT_CM = 80           # Height of camera from ground (cm)

FOCAL_LENGTH_PIXELS = 800       # Focal length in pixels

SENSOR_WIDTH_MM = 36            # Sensor width in mm

REFERENCE_OBJECT_SIZE_CM = 30   # Known reference (e.g., road marking width in cm)

# Baseline pixel-to-cm scale used when no reference object is supplied.

# Assumes a 640 px wide image of a pothole captured from ~1-1.5 m away,

# covering roughly 220 cm of ground (~0.35 cm per pixel).

#

# WARNING: This is an ESTIMATE ONLY. Real-world accuracy requires

# calibration with a reference object of known size. Adjust this value

# for your specific camera setup and mounting height.

PIXEL_TO_CM_BASELINE = 0.35

# Preprocessing settings

CLAHE_CLIP_LIMIT = 3.0          # Contrast enhancement clip limit

CLAHE_TILE_GRID_SIZE = (8, 8)   # CLAHE tile grid size

GAUSSIAN_BLUR_KERNEL = (3, 3)   # Gaussian blur kernel size

# ============================================================

# Depth Estimation Settings

# ============================================================

# MiDaS model settings

MIDAS_MODEL_TYPE = "DPT_Hybrid"  # "DPT_Hybrid", "DPT_Large", "DPT_Small"

MIDAS_TARGET_SIZE = (384, 384)   # Input size for MiDaS

# Depth range for normalization (in meters, relative)

MIN_DEPTH = 0.1

MAX_DEPTH = 10.0

# ============================================================

# Severity Classification Thresholds

# ============================================================

# Size thresholds in cm²

SIZE_SMALL_MAX_CM2 = 500        # < 500 cm² → Small

SIZE_MEDIUM_MAX_CM2 = 2000      # 500–2000 cm² → Medium

# > 2000 cm² → Large

# Depth thresholds in cm (estimated)

DEPTH_SHALLOW_MAX_CM = 2.0      # < 2 cm → Shallow

DEPTH_MODERATE_MAX_CM = 5.0     # 2–5 cm → Moderate

# > 5 cm → Deep

# Upper bound used to map normalized relative-depth differences to a

# plausible physical pothole depth in cm. Calibrate against measured

# ground truth for survey-grade accuracy.

DEPTH_MAX_CM_ESTIMATE = 25.0

# Severity priority mapping

SEVERITY_LEVELS = {

"Critical": {"color": "red", "priority": 1, "response_time_hours": 24},

"High": {"color": "orange", "priority": 2, "response_time_hours": 168},

"Medium": {"color": "yellow", "priority": 3, "response_time_hours": 720},

"Low": {"color": "green", "priority": 4, "response_time_hours": 2160},

}

# Size categories

SIZE_CATEGORIES = {

"Small": {"min": 0, "max": SIZE_SMALL_MAX_CM2},

"Medium": {"min": SIZE_SMALL_MAX_CM2, "max": SIZE_MEDIUM_MAX_CM2},

"Large": {"min": SIZE_MEDIUM_MAX_CM2, "max": float("inf")},

}

# Depth categories

DEPTH_CATEGORIES = {

"Shallow": {"min": 0, "max": DEPTH_SHALLOW_MAX_CM},

"Moderate": {"min": DEPTH_SHALLOW_MAX_CM, "max": DEPTH_MODERATE_MAX_CM},

"Deep": {"min": DEPTH_MODERATE_MAX_CM, "max": float("inf")},

}

# ============================================================

# API Settings

# ============================================================

API_HOST = os.environ.get("HOST", "0.0.0.0")

# Render / Railway / Heroku inject $PORT at runtime; fall back to 5000 locally.

API_PORT = int(os.environ.get("PORT", "5000"))

# Debug must be OFF in production (never expose the Werkzeug debugger).

# Set API_DEBUG=true explicitly for local development only.

API_DEBUG = os.environ.get("API_DEBUG", "false").lower() == "true"

# Maximum upload size (MB)

MAX_CONTENT_LENGTH_MB = 50

# Allowed file extensions

ALLOWED_EXTENSIONS = {"png", "jpg", "jpeg", "gif", "bmp", "tiff"}

# ============================================================

# Web Frontend Settings

# ============================================================

WEB_HOST = "127.0.0.1"

WEB_PORT = 5000

# Mapbox / Leaflet settings

MAP_DEFAULT_LAT = 37.7749    # Default center latitude

MAP_DEFAULT_LNG = -122.4194  # Default center longitude

MAP_DEFAULT_ZOOM = 15

# ============================================================

# Flask Secret Key (for session management)

# ============================================================

SECRET_KEY = os.environ.get("FLASK_SECRET_KEY", "pothole-detection-secret-key-2024")

# ============================================================

# Database Settings (SQLite for development)

# ============================================================

DATABASE_PATH = os.path.join(PROJECT_ROOT, "data", "pothole_reports.db")

SQLALCHEMY_DATABASE_URI = f"sqlite:///{DATABASE_PATH}"

SQLALCHEMY_TRACK_MODIFICATIONS = False

# ============================================================

# Logging Settings

# ============================================================

LOG_LEVEL = "INFO"

LOG_FILE = os.path.join(PROJECT_ROOT, "logs", "app.log")

# ============================================================

# Utility Functions

# ============================================================

def ensure_directories():

"""Create all required directories if they don't exist."""

for d in [DATA_DIR, MODELS_DIR, SAMPLES_DIR, UPLOADS_DIR, os.path.dirname(LOG_FILE)]:

os.makedirs(d, exist_ok=True)

# Ensure directories exist on import

ensure_directories()

# main.py

MAIN PIPELINE - detect -> size -> depth -> severity -> report

"""

Main Pipeline Orchestrator

==========================

Ties together detection, depth estimation, size estimation,

and severity classification into a single pipeline.

Usage:

python main.py --image road.jpg

python main.py --batch data/samples/

python main.py --web

"""

import os

import sys

import argparse

import json

import cv2

import numpy as np

from typing import List, Dict, Tuple, Optional

from datetime import datetime

from glob import glob

import config

from src.detector import PotholeDetector, DetectionResult

from src.depth_estimator import DepthEstimator, DepthResult

from src.size_estimator import SizeEstimator, SizeResult

from src.severity import SeverityClassifier, SeverityResult

from src.utils import setup_logger, save_json, load_image, draw_bounding_boxes

logger = setup_logger("main", config.LOG_FILE)

class PotholeAnalysisPipeline:

"""

Complete pipeline: detect -> estimate size -> estimate depth -> classify severity.

This orchestrator combines all sub-modules into a single end-to-end system.

"""

def __init__(self,

detector: PotholeDetector = None,

depth_estimator: DepthEstimator = None,

size_estimator: SizeEstimator = None,

severity_classifier: SeverityClassifier = None,

device: str = None):

"""

Initialize the analysis pipeline.

Args:

detector: PotholeDetector instance (creates default if None)

depth_estimator: DepthEstimator instance

size_estimator: SizeEstimator instance

severity_classifier: SeverityClassifier instance

device: Device for inference

"""

self.device = device or config.DEFAULT_DEVICE

self.size_estimator = size_estimator or SizeEstimator()

self.severity_classifier = severity_classifier or SeverityClassifier()

# Detector: prefer YOLOv8, but fall back to the classical detector

# when PyTorch/Ultralytics are unavailable (see config.ALLOW_CLASSICAL_FALLBACK).

self.detection_backend = "yolo"

if detector is not None:

self.detector = detector

else:

try:

self.detector = PotholeDetector(device=self.device)

except Exception as e:

if not getattr(config, "ALLOW_CLASSICAL_FALLBACK", True):

raise

from src.classical_detector import ClassicalPotholeDetector

logger.warning(

f"YOLOv8 unavailable ({e}); "

f"using ClassicalPotholeDetector fallback."

)

self.detector = ClassicalPotholeDetector()

self.detection_backend = "classical"

# Depth: prefer MiDaS, but synthesized brightness-based depth is used

# inside DepthEstimator when its torch model cannot load.

if depth_estimator is not None:

self.depth_estimator = depth_estimator

else:

try:

self.depth_estimator = DepthEstimator(device=self.device)

except Exception as e:

logger.warning(

f"MiDaS unavailable ({e}); depth will use the "

f"synthetic fallback inside DepthEstimator."

)

self.depth_estimator = DepthEstimator.__new__(DepthEstimator)

self.depth_estimator.model = None

self.depth_estimator.model_type = "synthetic-fallback"

self.depth_estimator.device = self.device

self.depth_backend = getattr(self.depth_estimator, "model_type", "unknown")

logger.info(

f"PotholeAnalysisPipeline initialized "

f"(detector={self.detection_backend}, depth={self.depth_backend})"

)

def analyze_image(self, image: np.ndarray,

gps_coords: Tuple[float, float] = None,

reference_bbox: Tuple[int, int, int, int] = None,

visualize: bool = True) -> Dict:

"""

Run the complete analysis pipeline on a single image.

Args:

image: Input road image (BGR format)

gps_coords: Optional GPS coordinates (lat, lng)

reference_bbox: Optional reference object bbox for calibration

visualize: Whether to draw results on the image

Returns:

Dictionary containing:

- detections: List of enriched detection results

- annotated_image: Image with visualizations (if visualize=True)

- report: Complete analysis report

"""

start_time = datetime.now()

img_h, img_w = image.shape[:2]

# Step 1: Detect potholes

logger.info("Step 1: Running pothole detection...")

results = self.detector.detect(image)

raw_detections = self.detector.extract_detections(results)

logger.info(f"  Found {len(raw_detections)} pothole(s)")

# Step 2: Estimate size for each detection

logger.info("Step 2: Estimating pothole sizes...")

for det in raw_detections:

size_result = self.size_estimator.estimate_size(

det.bbox, img_w, reference_bbox

)

det.width_cm = size_result.width_cm

det.height_cm = size_result.height_cm

det.size_cm2 = size_result.area_cm2

det.size_category = size_result.size_category

# Step 3: Estimate depth for each detection

logger.info("Step 3: Estimating pothole depths...")

depth_map = self.depth_estimator.estimate_depth(image)

for det in raw_detections:

depth_cm, depth_cat = self.depth_estimator.get_pothole_depth(

depth_map, det.bbox

)

det.depth_cm = depth_cm

det.depth_category = depth_cat

# Step 4: Classify severity

logger.info("Step 4: Classifying severity...")

for det in raw_detections:

severity_result = self.severity_classifier.classify(

area_cm2=det.size_cm2,

size_category=det.size_category,

depth_cm=det.depth_cm,

depth_category=det.depth_category,

)

det.severity = severity_result.severity

det.priority = severity_result.priority

det.response_time_hours = severity_result.response_time_hours

# Step 5: Visualize

annotated_image = None

if visualize:

annotated_image = self._annotate_image(image, raw_detections, depth_map)

elapsed = (datetime.now() - start_time).total_seconds()

# Generate report

report = self._generate_report(

raw_detections, image, gps_coords, elapsed

)

logger.info(f"Pipeline complete in {elapsed:.2f}s")

return {

"detections": [d.to_dict() for d in raw_detections],

"annotated_image": annotated_image,

"report": report,

"processing_time_seconds": elapsed,

}

def _annotate_image(self, image: np.ndarray,

detections: List[DetectionResult],

depth_map: np.ndarray) -> np.ndarray:

"""Create annotated output image with boxes, labels, and severity colors."""

from src.utils import draw_bounding_boxes as original_draw

# Use custom drawing with severity colors

img = image.copy()

for det in detections:

x1, y1, x2, y2 = det.bbox

color = det.get_color() if hasattr(det, 'get_color') else (0, 255, 0)

# Use severity color

from src.severity import SeverityClassifier

temp_result = self.severity_classifier.classify(

area_cm2=det.size_cm2,

size_category=det.size_category,

depth_cm=det.depth_cm,

depth_category=det.depth_category,

)

color = temp_result.color_bgr

# Draw bounding box

cv2.rectangle(img, (x1, y1), (x2, y2), color, 2)

# Draw label

label = f"Pothole: {det.confidence:.2f} | "

label += f"{det.size_cm2:.0f}cm2 ({det.size_category}) | "

label += f"D:{det.depth_cm:.1f}cm ({det.depth_category}) | "

label += f"{det.severity}"

(w, h), _ = cv2.getTextSize(label, cv2.FONT_HERSHEY_SIMPLEX, 0.4, 1)

cv2.rectangle(img, (x1, y1 - h - 4), (x1 + w, y1), color, -1)

cv2.putText(img, label, (x1, y1 - 2),

cv2.FONT_HERSHEY_SIMPLEX, 0.4, (255, 255, 255), 1)

return img

def _generate_report(self, detections: List[DetectionResult],

image: np.ndarray,

gps_coords: Tuple[float, float],

elapsed: float) -> Dict:

"""Generate a complete analysis report."""

detection_dicts = [d.to_dict() for d in detections]

critical = sum(1 for d in detections if d.severity == "Critical")

high = sum(1 for d in detections if d.severity == "High")

medium = sum(1 for d in detections if d.severity == "Medium")

low = sum(1 for d in detections if d.severity == "Low")

report = {

"timestamp": datetime.now().isoformat(),

"gps_coordinates": {"lat": gps_coords[0], "lng": gps_coords[1]} if gps_coords else None,

"image_shape": {"height": image.shape[0], "width": image.shape[1]},

"total_potholes": len(detections),

"severity_summary": {

"critical": critical,

"high": high,

"medium": medium,

"low": low,

},

"potholes": detection_dicts,

"processing_time_seconds": round(elapsed, 3),

"model_info": {

"detector": getattr(self, "detection_backend", "yolo"),

"depth_model": getattr(self, "depth_backend",

"MiDaS DPT_Hybrid"),

"device": self.device,

},

}

return report

def analyze_from_path(self, image_path: str,

gps_coords: Tuple[float, float] = None,

output_dir: str = None) -> Dict:

"""Analyze an image from file path and optionally save results."""

image = load_image(image_path)

result = self.analyze_image(image, gps_coords=gps_coords)

if output_dir:

os.makedirs(output_dir, exist_ok=True)

filename = os.path.splitext(os.path.basename(image_path))[0]

# Save annotated image

annotated_path = os.path.join(output_dir, f"{filename}_annotated.jpg")

cv2.imwrite(annotated_path, result["annotated_image"])

# Save report

report_path = os.path.join(output_dir, f"{filename}_report.json")

save_json(result["report"], report_path)

result["annotated_image_path"] = annotated_path

result["report_path"] = report_path

return result

def process_folder(self, folder_path: str,

gps_coords: Tuple[float, float] = None,

output_dir: str = None,

pattern: str = "*.*") -> List[Dict]:

"""Process all images in a folder."""

image_extensions = {".jpg", ".jpeg", ".png", ".bmp", ".tiff", ".gif"}

files = glob(os.path.join(folder_path, "**", pattern), recursive=True)

image_files = [f for f in files

if os.path.splitext(f)[1].lower() in image_extensions]

results = []

for image_path in image_files:

logger.info(f"Processing: {image_path}")

try:

result = self.analyze_from_path(image_path, gps_coords, output_dir)

results.append(result)

except Exception as e:

logger.error(f"Failed to process {image_path}: {e}")

return results

def main():

"""Command-line entry point."""

parser = argparse.ArgumentParser(

description="Road Pothole Detection and Severity Analysis"

)

parser.add_argument("--image", type=str, default=None,

help="Path to input image")

parser.add_argument("--batch", type=str, default=None,

help="Path to folder for batch processing")

parser.add_argument("--output", type=str, default="outputs",

help="Output directory for results")

parser.add_argument("--gps-lat", type=float, default=None,

help="GPS latitude")

parser.add_argument("--gps-lng", type=float, default=None,

help="GPS longitude")

parser.add_argument("--web", action="store_true",

help="Start web API server")

args = parser.parse_args()

if args.web:

# Start Flask web server

from api.app import app

app.run(host=config.API_HOST, port=config.API_PORT, debug=config.API_DEBUG)

return

# Initialize pipeline

logger.info("Initializing Pothole Analysis Pipeline...")

pipeline = PotholeAnalysisPipeline()

gps = None

if args.gps_lat and args.gps_lng:

gps = (args.gps_lat, args.gps_lng)

if args.image:

# Single image processing

result = pipeline.analyze_from_path(args.image, gps, args.output)

print(f"\n{'='*60}")

print(f"Results for: {args.image}")

print(f"{'='*60}")

for det in result["detections"]:

print(f"  Pothole: {det['confidence']:.2f} conf")

print(f"  Size: {det['size_cm2']} cm2 ({det['size_category']})")

print(f"  Depth: {det['depth_cm']} cm ({det['depth_category']})")

print(f"  Severity: {det['severity']} (Priority {det['priority']})")

print(f"  Response: {det['response_time_hours']}h")

print(f"\nProcessing time: {result['processing_time_seconds']:.2f}s")

print(f"Annotated image: {result.get('annotated_image_path', 'N/A')}")

print(f"Report: {result.get('report_path', 'N/A')}")

elif args.batch:

results = pipeline.process_folder(args.batch, gps, args.output)

print(f"\nProcessed {len(results)} images successfully.")

else:

print("No input specified. Use --image or --batch.")

parser.print_help()

if __name__ == "__main__":

main()

# src/detector.py

YOLOv8 detector + DetectionResult data structure (primary backend)

"""

Pothole Detection Module

========================

Uses YOLOv8 (Ultralytics) for real-time pothole detection from road images.

Usage:

from src.detector import PotholeDetector

detector = PotholeDetector(model_path="data/models/best.pt")

results = detector.detect(image)

detections = detector.extract_detections(results)

"""

import os

import cv2

import numpy as np

from typing import List, Dict, Tuple, Optional, Any

from dataclasses import dataclass

def _torch_usable():

"""Return True only if torch is installed AND its native libs load.

The probe runs in a subprocess because a broken torch install can raise

beyond ImportError/OSError (segfault / access violation), which would

otherwise crash this interpreter during a plain `import torch`.

"""

try:

from importlib import metadata as _metadata

_metadata.version("torch")

except Exception:

return False

try:

import subprocess as _sp

import sys as _sys

r = _sp.run(

[_sys.executable, "-c",

"import torch; "

"assert hasattr(torch, 'cuda'); "

"torch.cuda.is_available()"],

capture_output=True, timeout=120,

)

return r.returncode == 0

except Exception:

return False

try:

_TORCH_OK = _torch_usable()

except Exception:

_TORCH_OK = False

if _TORCH_OK:

import torch

from ultralytics import YOLO

TORCH_AVAILABLE = True

else:

TORCH_AVAILABLE = False

import config

from . import utils

from . import preprocessing

from .utils import setup_logger

logger = setup_logger(__name__)

@dataclass

class DetectionResult:

"""Data class representing a single pothole detection."""

bbox: List[int]

confidence: float

class_id: int

class_name: str

width_px: int

height_px: int

center: Tuple[int, int]

size_cm2: float = 0.0

size_category: str = ""

width_cm: float = 0.0

height_cm: float = 0.0

depth_cm: float = 0.0

depth_category: str = ""

severity: str = ""

priority: int = 4

response_time_hours: int = 0

def to_dict(self) -> Dict[str, Any]:

return {

"bbox": self.bbox,

"confidence": round(self.confidence, 4),

"class_id": self.class_id,

"class_name": self.class_name,

"width_px": self.width_px,

"height_px": self.height_px,

"center": list(self.center),

"size_cm2": round(self.size_cm2, 2),

"size_category": self.size_category,

"width_cm": round(self.width_cm, 2),

"height_cm": round(self.height_cm, 2),

"depth_cm": round(self.depth_cm, 2),

"depth_category": self.depth_category,

"severity": self.severity,

"priority": self.priority,

"response_time_hours": self.response_time_hours,

}

class PotholeDetector:

"""Pothole detection using YOLOv8."""

def __init__(self, model_path=None, device=None,

conf_thres=None, iou_thres=None):

if not TORCH_AVAILABLE:

raise ImportError(

"PyTorch and Ultralytics YOLO are required. "

"Install with: pip install torch ultralytics"

)

if model_path is None:

model_path = config.YOLO_MODEL_PATH

self.conf_thres = conf_thres or config.CONFIDENCE_THRESHOLD

self.iou_thres = iou_thres or config.IOU_THRESHOLD

self.device = device or config.DEFAULT_DEVICE

self.model = self._load_model(model_path)

self.model.to(self.device)

logger.info(f"PotholeDetector initialized on device: {self.device}")

def _load_model(self, model_path):

if os.path.exists(model_path):

logger.info(f"Loading custom model from: {model_path}")

return YOLO(model_path)

for name in ["yolov8n.pt", "yolov8s.pt", "yolov8m.pt"]:

try:

logger.info(f"Trying model: {name}")

return YOLO(name)

except Exception:

continue

raise FileNotFoundError(

f"Model not found at {model_path}. "

"Please download a YOLOv8 model or provide a valid model path."

)

def detect(self, image, preprocess_input=False):

if preprocess_input:

img = preprocessing.enhance_contrast(image)

else:

img = image

results = self.model(

img, conf=self.conf_thres, iou=self.iou_thres,

verbose=False, device=self.device,

)

return results[0] if len(results) > 0 else results

def detect_and_extract(self, image):

results = self.detect(image)

return self.extract_detections(results)

def detect_from_path(self, image_path):

image = utils.load_image(image_path)

detections = self.detect_and_extract(image)

return image, detections

def extract_detections(self, results):

"""Extract structured detection results from YOLO output."""

detections = []

if results.boxes is None or len(results.boxes) == 0:

return detections

if hasattr(results, 'orig_shape'):

image_shape = results.orig_shape

else:

image_shape = (720, 1280)

img_h, img_w = image_shape[:2]

for box in results.boxes:

xyxy = box.xyxy[0].cpu().numpy()

conf = float(box.conf.cpu().numpy()[0])

cls_id = int(box.cls.cpu().numpy()[0])

x1, y1, x2, y2 = xyxy

x1, y1, x2, y2 = int(x1), int(y1), int(x2), int(y2)

x1, y1 = max(0, x1), max(0, y1)

x2, y2 = min(img_w - 1, x2), min(img_h - 1, y2)

width_px = x2 - x1

height_px = y2 - y1

class_name = config.CLASS_NAMES.get(cls_id, f"class_{cls_id}")

detection = DetectionResult(

bbox=[x1, y1, x2, y2], confidence=conf, class_id=cls_id,

class_name=class_name, width_px=width_px,

height_px=height_px,

center=((x1 + x2) // 2, (y1 + y2) // 2),

)

detections.append(detection)

logger.info(f"Extracted {len(detections)} detections")

return detections

def visualize_detections(self, image, detections, show_labels=True):

"""Draw detection results on the image."""

img = image.copy()

for det in detections:

x1, y1, x2, y2 = det.bbox

color = (0, 255, 0)

cv2.rectangle(img, (x1, y1), (x2, y2), color, 2)

if show_labels:

label = f"Pothole: {det.confidence:.2f}"

(w, h), _ = cv2.getTextSize(

label, cv2.FONT_HERSHEY_SIMPLEX, 0.5, 1)

cv2.rectangle(img, (x1, y1 - h - 4),

(x1 + w, y1), color, -1)

cv2.putText(img, label, (x1, y1 - 2),

cv2.FONT_HERSHEY_SIMPLEX, 0.5,

(0, 0, 0), 1)

return img

def get_model_info(self):

return {

"model_path": config.YOLO_MODEL_PATH,

"device": self.device,

"conf_threshold": self.conf_thres,

"iou_threshold": self.iou_thres,

"class_names": config.CLASS_NAMES,

}

# src/classical_detector.py

Classical CV detector: Otsu+adaptive threshold, morphology, contours

"""

Classical Computer-Vision Pothole Detector (no deep-learning dependency)

========================================================================

Fallback detector using traditional image processing:

1. Grayscale + CLAHE contrast enhancement

2. GLOBAL (Otsu) + LOCAL (adaptive) thresholding combined - Otsu catches

the full body of large dark blobs, adaptive catches edges/subtle

regions; neither alone is sufficient

3. Brightness verification against a LARGE-window local mean (window

scales with image size) so big uniform potholes are not rejected

4. Morphological open/close/dilate clean-up

5. Contour extraction with shape/area filtering

6. Nearby-box merging so one pothole fragmented into several contours

reports as a single bounding box

Usage:

from src.classical_detector import ClassicalPotholeDetector

detector = ClassicalPotholeDetector()

detections = detector.detect_and_extract(image)

"""

import cv2

import numpy as np

from typing import List, Tuple, Optional

import config

from .utils import setup_logger

logger = setup_logger(__name__)

class ClassicalPotholeDetector:

"""

Pothole detector using classical image processing.

Detects potholes as dark, roughly elliptical regions on the road surface.

Suitable as a CPU-only, dependency-light fallback to YOLOv8.

Attributes:

min_area_px: Minimum contour area (pixels) to count as a pothole

max_area_ratio: Maximum contour area as a fraction of the image

darkness_threshold: How much darker than the local mean a region must be

merge_gap_px: Max pixel gap between fragments merged into one pothole

"""

def __init__(self,

min_area_px: int = 300,

max_area_ratio: float = 0.30,

darkness_threshold: float = 0.80,

block_size: int = 51,

constant_c: int = 10,

merge_gap_px: int = 40):

self.min_area_px = min_area_px

self.max_area_ratio = max_area_ratio

self.darkness_threshold = darkness_threshold

self.block_size = block_size if block_size % 2 == 1 else block_size + 1

self.constant_c = constant_c

self.merge_gap_px = merge_gap_px

logger.info(

f"ClassicalPotholeDetector initialized "

f"(min_area={min_area_px}px, block={self.block_size}, "

f"C={constant_c}, merge_gap={merge_gap_px}px)"

)

def get_model_info(self) -> dict:

"""Return detector information (mirrors PotholeDetector API)."""

return {

"model_path": "classical-cv (no weights)",

"device": "cpu",

"conf_threshold": self.darkness_threshold,

"iou_threshold": None,

"class_names": config.CLASS_NAMES,

"method": "otsu+adaptive threshold + morphology + contours",

}

def detect(self, image: np.ndarray, preprocess_input: bool = False):

"""

Run classical detection.

Args:

image: BGR image

preprocess_input: Accepted for API compatibility (unused)

Returns:

List of (contour_or_None, bbox, score, area) tuples

"""

return self._find_pothole_contours(image)

def _find_pothole_contours(self, image: np.ndarray):

"""Core classical detection routine."""

img_h, img_w = image.shape[:2]

img_area = float(img_h * img_w)

# 1. Grayscale + contrast enhancement

gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

clahe = cv2.createCLAHE(clipLimit=config.CLAHE_CLIP_LIMIT,

tileGridSize=config.CLAHE_TILE_GRID_SIZE)

gray = clahe.apply(gray)

# 2. Mild blur to suppress texture noise

blurred = cv2.GaussianBlur(gray, (7, 7), 0)

# 3a. GLOBAL threshold (Otsu): captures the FULL body of large

#     uniform dark blobs, which adaptive thresholding alone misses

#     (inside a blob bigger than the window, the local neighborhood

#     is equally dark so nothing looks "darker than local mean").

_, otsu_mask = cv2.threshold(

blurred, 0, 255,

cv2.THRESH_BINARY_INV + cv2.THRESH_OTSU)

# 3b. LOCAL adaptive threshold: catches edges and subtle regions

adaptive_mask = cv2.adaptiveThreshold(

blurred, 255,

cv2.ADAPTIVE_THRESH_GAUSSIAN_C,

cv2.THRESH_BINARY_INV,

self.block_size,

self.constant_c,

)

combined = cv2.bitwise_or(otsu_mask, adaptive_mask)

# 4. Morphological clean-up + slight dilation to bridge gaps

kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (9, 9))

combined = cv2.morphologyEx(combined, cv2.MORPH_OPEN, kernel,

iterations=2)

combined = cv2.morphologyEx(combined, cv2.MORPH_CLOSE, kernel,

iterations=3)

# 5. Brightness verification against a LARGE-window local mean.

#    The window must be much bigger than the biggest expected

#    pothole (~1/3 of image dims) so the interior of a large blob

#    is still "darker than its surroundings".

win = max(151, (max(img_h, img_w) // 3) | 1)  # force odd

local_mean = cv2.blur(blurred.astype(np.float32), (win, win))

brightness_ok = (blurred.astype(np.float32) <

local_mean * self.darkness_threshold)

mask = cv2.bitwise_and(combined,

(brightness_ok * 255).astype(np.uint8))

# 6. Extract contours

contours, _ = cv2.findContours(mask, cv2.RETR_EXTERNAL,

cv2.CHAIN_APPROX_SIMPLE)

results = []

for cnt in contours:

area = cv2.contourArea(cnt)

if area < self.min_area_px or area > img_area * self.max_area_ratio:

continue

x, y, w, h = cv2.boundingRect(cnt)

if h == 0 or w == 0:

continue

# Shape filters: potholes are roughly elliptical, not thin lines

aspect = w / float(h)

if aspect < 0.25 or aspect > 4.0:

continue

# Circularity: 4*pi*area / perimeter^2

perimeter = cv2.arcLength(cnt, True)

if perimeter <= 0:

continue

circularity = 4 * np.pi * area / (perimeter * perimeter)

if circularity < 0.15:

continue

# Fill ratio: how much of the bbox the blob occupies

fill_ratio = area / float(w * h)

if fill_ratio < 0.30:

continue

# Confidence: combination of circularity, fill and darkness

region = gray[y:y + h, x:x + w]

local_ref = local_mean[y:y + h, x:x + w]

darkness = 1.0 - (float(np.mean(region)) /

max(float(np.mean(local_ref)), 1.0))

darkness = float(np.clip(darkness * 3.0, 0.0, 1.0))

score = float(np.clip(

0.4 * min(circularity / 0.8, 1.0) +

0.3 * min(fill_ratio / 0.7, 1.0) +

0.3 * darkness,

0.05, 0.99,

))

results.append((cnt, [x, y, x + w, y + h], score, area))

# 7. Merge nearby fragments into single pothole boxes

results = self._merge_results(results, self.merge_gap_px)

return results

@staticmethod

def _merge_results(results, gap_px: int = 40):

"""

Merge detection fragments whose bounding boxes are within `gap_px`

of each other into a single detection (union box, max score,

summed area). A single pothole often fragments into several

contours after thresholding.

"""

if not results:

return results

boxes = [list(r[1]) for r in results]

scores = [r[2] for r in results]

areas = [r[3] for r in results]

used = [False] * len(boxes)

merged = []

for i in range(len(boxes)):

if used[i]:

continue

x1, y1, x2, y2 = boxes[i]

best = scores[i]

tot = areas[i]

used[i] = True

changed = True

while changed:

changed = False

for j in range(len(boxes)):

if used[j]:

continue

bx1, by1, bx2, by2 = boxes[j]

if not (bx1 - gap_px > x2 or bx2 + gap_px < x1 or

by1 - gap_px > y2 or by2 + gap_px < y1):

x1 = min(x1, bx1)

y1 = min(y1, by1)

x2 = max(x2, bx2)

y2 = max(y2, by2)

best = max(best, scores[j])

tot += areas[j]

used[j] = True

changed = True

merged.append((None, [x1, y1, x2, y2], best, tot))

merged.sort(key=lambda r: r[3], reverse=True)

return merged

def extract_detections(self, classical_results):

"""Convert classical results into DetectionResult objects."""

from .detector import DetectionResult

detections: List = []

for _cnt, bbox, score, _area in classical_results:

x1, y1, x2, y2 = bbox

detections.append(DetectionResult(

bbox=[x1, y1, x2, y2],

confidence=round(score, 4),

class_id=0,

class_name=config.CLASS_NAMES.get(0, "Pothole"),

width_px=x2 - x1,

height_px=y2 - y1,

center=((x1 + x2) // 2, (y1 + y2) // 2),

))

logger.info(f"Classical detector found {len(detections)} candidate(s)")

return detections

def detect_and_extract(self, image: np.ndarray):

"""Convenience: detect + convert in one step."""

return self.extract_detections(self.detect(image))

def detect_from_path(self, image_path: str):

"""Load image from disk, detect, and return (image, detections)."""

from .utils import load_image

image = load_image(image_path)

return image, self.detect_and_extract(image)

def visualize_detections(self, image: np.ndarray, detections,

show_labels: bool = True) -> np.ndarray:

"""Draw detections (mirrors PotholeDetector.visualize_detections)."""

img = image.copy()

for det in detections:

x1, y1, x2, y2 = det.bbox

color = (0, 255, 0)

cv2.rectangle(img, (x1, y1), (x2, y2), color, 2)

if show_labels:

label = f"Pothole: {det.confidence:.2f}"

(w, h), _ = cv2.getTextSize(label,

cv2.FONT_HERSHEY_SIMPLEX, 0.5, 1)

cv2.rectangle(img, (x1, y1 - h - 4), (x1 + w, y1), color, -1)

cv2.putText(img, label, (x1, y1 - 2),

cv2.FONT_HERSHEY_SIMPLEX, 0.5, (0, 0, 0), 1)

return img

# src/preprocessing.py

Image preprocessing: CLAHE contrast, denoise, lighting/shadow removal

"""

Image Preprocessing Module

============================

Handles image enhancement, normalization, and preparation

for deep learning models.

"""

import cv2

import numpy as np

from typing import Tuple, Optional

import config

def enhance_contrast(image: np.ndarray,

clip_limit: float = None,

tile_grid_size: Tuple[int, int] = None) -> np.ndarray:

"""

Enhance image contrast using CLAHE.

Args:

image: Input image (BGR format)

clip_limit: CLAHE clip limit

tile_grid_size: Tile grid size for CLAHE

Returns:

Contrast-enhanced image

"""

if clip_limit is None:

clip_limit = config.CLAHE_CLIP_LIMIT

if tile_grid_size is None:

tile_grid_size = config.CLAHE_TILE_GRID_SIZE

lab = cv2.cvtColor(image, cv2.COLOR_BGR2LAB)

l_channel, a, b = cv2.split(lab)

clahe = cv2.createCLAHE(clipLimit=clip_limit, tileGridSize=tile_grid_size)

l_channel = clahe.apply(l_channel)

limg = cv2.merge((l_channel, a, b))

enhanced = cv2.cvtColor(limg, cv2.COLOR_LAB2BGR)

return enhanced

def reduce_noise(image: np.ndarray,

blur_kernel: Tuple[int, int] = None,

bilateral_iterations: int = 1) -> np.ndarray:

"""Reduce image noise using Gaussian blur and bilateral filtering."""

if blur_kernel is None:

blur_kernel = config.GAUSSIAN_BLUR_KERNEL

denoised = cv2.GaussianBlur(image, blur_kernel, 0)

for _ in range(bilateral_iterations):

denoised = cv2.bilateralFilter(denoised, 9, 75, 75)

return denoised

def normalize_image(image: np.ndarray,

mean: Tuple[float, float, float] = (0.485, 0.456, 0.406),

std: Tuple[float, float, float] = (0.229, 0.224, 0.225)) -> np.ndarray:

"""

Normalize image pixel values using ImageNet statistics.

Args:

image: Input image (uint8)

mean: Per-channel mean values

std: Per-channel standard deviation values

Returns:

Normalized image (float32)

"""

image_rgb = cv2.cvtColor(image, cv2.COLOR_BGR2RGB)

image_normalized = image_rgb.astype(np.float32) / 255.0

for i in range(3):

image_normalized[:, :, i] = (image_normalized[:, :, i] - mean[i]) / std[i]

return image_normalized

def correct_lighting(image: np.ndarray) -> np.ndarray:

"""Correct non-uniform lighting using background estimation."""

gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (101, 101))

bg = cv2.morphologyEx(gray, cv2.MORPH_OPEN, kernel)

bg = cv2.GaussianBlur(bg, (101, 101), 0)

bg = np.maximum(bg, 1)

result = image.copy().astype(np.float32)

bg_3ch = np.stack([bg, bg, bg], axis=2)

result = result / bg_3ch * 128.0

result = np.clip(result, 0, 255).astype(np.uint8)

return result

def remove_shadow(image: np.ndarray) -> np.ndarray:

"""Reduce shadow effects in the image."""

hsv = cv2.cvtColor(image, cv2.COLOR_BGR2HSV)

h, s, v = cv2.split(hsv)

clahe = cv2.createCLAHE(clipLimit=2.0, tileGridSize=(8, 8))

v = clahe.apply(v)

hsv = cv2.merge([h, s, v])

return cv2.cvtColor(hsv, cv2.COLOR_HSV2BGR)

def preprocess_image(image: np.ndarray,

normalize: bool = True,

target_size: Tuple[int, int] = None,

enhance: bool = True,

denoise: bool = True,

correct_light: bool = True) -> np.ndarray:

"""

Full preprocessing pipeline for road images.

Args:

image: Input image (BGR format)

normalize: Whether to normalize pixel values

target_size: Optional (width, height) to resize to

enhance: Apply CLAHE contrast enhancement

denoise: Apply noise reduction

correct_light: Apply lighting correction

Returns:

Preprocessed image

"""

img = image.copy()

if correct_light:

img = correct_lighting(img)

if enhance:

img = enhance_contrast(img)

if denoise:

img = reduce_noise(img, bilateral_iterations=1)

img = remove_shadow(img)

if target_size is not None:

img = cv2.resize(img, target_size, interpolation=cv2.INTER_AREA)

if normalize:

img = normalize_image(img)

return img

def extract_pothole_region(image: np.ndarray,

bbox: Tuple[int, int, int, int],

padding: int = 10) -> np.ndarray:

"""

Extract and preprocess a pothole region from the image.

Args:

image: Full road image

bbox: (x1, y1, x2, y2) bounding box coordinates

padding: Extra pixels around the bounding box

Returns:

Cropped and preprocessed pothole region

"""

x1, y1, x2, y2 = bbox

h, w = image.shape[:2]

x1 = max(0, x1 - padding)

y1 = max(0, y1 - padding)

x2 = min(w, x2 + padding)

y2 = min(h, y2 + padding)

region = image[y1:y2, x1:x2]

region = enhance_contrast(region)

region = reduce_noise(region, bilateral_iterations=1)

return region

# src/depth_estimator.py

Depth estimation: MiDaS DPT + synthetic fallback, depth categories

"""

Depth Estimation Module

=======================

Uses MiDaS (via PyTorch Hub) for monocular depth estimation

from pothole images.

Classes:

- DepthEstimator: Main depth estimation class

Usage:

from src.depth_estimator import DepthEstimator

estimator = DepthEstimator()

depth_map = estimator.estimate_depth(image)

pothole_depth = estimator.get_pothole_depth(depth_map, bbox)

"""

import os

import cv2

import numpy as np

from typing import Tuple, Optional

from dataclasses import dataclass

def _torch_usable():

"""Return True only if torch is installed AND its native libs load.

The probe runs in a subprocess because a broken torch install can raise

beyond ImportError/OSError (segfault / access violation), which would

otherwise crash this interpreter during a plain `import torch`.

"""

try:

from importlib import metadata as _metadata

_metadata.version("torch")

except Exception:

return False

try:

import subprocess as _sp

import sys as _sys

r = _sp.run(

[_sys.executable, "-c",

"import torch; "

"assert hasattr(torch, 'cuda'); "

"torch.cuda.is_available()"],

capture_output=True, timeout=120,

)

return r.returncode == 0

except Exception:

return False

try:

_TORCH_OK = _torch_usable()

except Exception:

_TORCH_OK = False

if _TORCH_OK:

import torch

import torch.nn.functional as F

TORCH_AVAILABLE = True

else:

TORCH_AVAILABLE = False

import config

from .utils import setup_logger

logger = setup_logger(__name__)

@dataclass

class DepthResult:

"""Container for depth estimation results."""

depth_map: np.ndarray       # Full image depth map (H, W)

relative_depth: np.ndarray  # Normalized depth [0, 1]

pothole_depth_cm: float = 0.0

depth_category: str = ""

depth_percentile: float = 0.0

class DepthEstimator:

"""

Monocular depth estimation using MiDaS.

Attributes:

model: MiDaS model instance

device: Device for inference

model_type: MiDaS model variant

"""

def __init__(self, model_type: str = None, device: str = None):

if not TORCH_AVAILABLE:

raise ImportError(

"PyTorch is required for depth estimation. "

"Install with: pip install torch"

)

self.model_type = model_type or config.MIDAS_MODEL_TYPE

self.device = device or config.DEFAULT_DEVICE

if self.model_type == "DPT_Hybrid":

model_repo = "isl-org/MiDaS"

model_name = "DPT_Hybrid"

elif self.model_type == "DPT_Large":

model_repo = "isl-org/MiDaS"

model_name = "DPT_Large"

else:

model_repo = "isl-org/MiDaS"

model_name = "DPT_Hybrid"

self._load_model(model_repo, model_name)

logger.info(f"DepthEstimator initialized with {self.model_type} on {self.device}")

def _load_model(self, repo: str, model_name: str):

"""Load MiDaS model from PyTorch Hub or cache."""

try:

self.model = torch.hub.load(repo, model_name, pretrained=True)

self.model.eval()

self.model.to(self.device)

# Load transforms

self.transforms = torch.hub.load(repo, "transforms")[0]

logger.info(f"MiDaS {model_name} loaded successfully")

except Exception as e:

logger.error(f"Failed to load MiDaS: {e}")

logger.info("Will use synthetic depth estimation as fallback")

self.model = None

def estimate_depth(self, image: np.ndarray) -> np.ndarray:

"""

Estimate depth map for an input image.

Args:

image: Input image (BGR format)

Returns:

Depth map (H, W) with normalized depth values

"""

if self.model is None:

return self._synthetic_depth(image)

# Convert BGR to RGB

img_rgb = cv2.cvtColor(image, cv2.COLOR_BGR2RGB)

# Apply MiDaS transforms

input_batch = self.transforms(img_rgb).unsqueeze(0).to(self.device)

with torch.no_grad():

depth = self.model(input_batch)

# Resize to original image size

depth = F.interpolate(

depth.unsqueeze(1),

size=img_rgb.shape[:2],

mode="bicubic",

align_corners=False,

).squeeze()

depth_map = depth.cpu().numpy()

return depth_map

def _synthetic_depth(self, image: np.ndarray) -> np.ndarray:

"""

Fallback: Generate a synthetic depth map based on brightness/shadows.

This is used when MiDaS is not available.

"""

gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

gray_f = gray.astype(np.float32) / 255.0

# Darker areas = deeper (shadows in potholes)

# Invert: brighter = closer, darker = farther

depth_map = 1.0 - gray_f

# Apply Gaussian blur to smooth

depth_map = cv2.GaussianBlur(depth_map, (15, 15), 0)

# Normalize to [0, 1]

depth_min = depth_map.min()

depth_max = depth_map.max()

if depth_max > depth_min:

depth_map = (depth_map - depth_min) / (depth_max - depth_min)

return depth_map.astype(np.float32)

def get_pothole_depth(self, depth_map: np.ndarray,

bbox: Tuple[int, int, int, int],

reference_depth: float = None) -> Tuple[float, str]:

"""

Estimate pothole depth from the depth map within the bounding box.

Args:

depth_map: Full image depth map

bbox: (x1, y1, x2, y2) bounding box

reference_depth: Depth of surrounding road (meters)

Returns:

Tuple of (depth_in_cm, depth_category)

"""

x1, y1, x2, y2 = bbox

region = depth_map[y1:y2, x1:x2]

if region.size == 0:

return 0.0, ""

# Pothole depth = surrounding road depth - pothole depth (inverted)

# Since MiDaS gives relative depth, deeper = higher value

# In road images, potholes appear darker/deeper

mean_depth = float(np.mean(region))

min_depth = float(np.min(region))

# Estimate depth difference from surrounding road

# Use a ring around the bounding box as reference

h, w = depth_map.shape

pad = 20

x1_ref = max(0, x1 - pad)

y1_ref = max(0, y1 - pad)

x2_ref = min(w, x2 + pad)

y2_ref = min(h, y2 + pad)

# Extract reference region (excluding pothole area)

ref_region = depth_map[y1_ref:y2_ref, x1_ref:x2_ref].copy()

ref_h, ref_w = ref_region.shape

# Mask out the pothole area from reference

pothole_h = y2 - y1

pothole_w = x2 - x1

py_offset = y1 - y1_ref

px_offset = x1 - x1_ref

if px_offset >= 0 and py_offset >= 0 and \

px_offset + pothole_w <= ref_w and py_offset + pothole_h <= ref_h:

ref_region[py_offset:py_offset + pothole_h,

px_offset:px_offset + pothole_w] = 0

ref_region = ref_region[ref_region > 0]

ref_mean = float(np.mean(ref_region)) if len(ref_region) > 0 else mean_depth

# Depth difference (pothole is deeper = lower relative depth value)

depth_diff = abs(ref_mean - mean_depth)

depth_diff = float(min(max(depth_diff, 0.0), 1.0))

# Map the normalized difference to a plausible physical pothole depth.

# Relative / synthetic depth maps are dimensionless, so this is a

# calibrated estimate: full contrast (1.0) -> 25 cm, which still lands

# in the Deep category but stays within realistic pothole geometry.

# NOTE: For survey-grade accuracy, calibrate this against measured

# ground-truth depths or use stereo/LiDAR (see docs).

max_physical_cm = getattr(config, "DEPTH_MAX_CM_ESTIMATE", 25.0)

depth_cm = depth_diff * max_physical_cm

# Geometric plausibility bound: a routine pothole is wider than it is

# deep. Cap the estimate at ~50% of the pothole's minor real-world

# dimension (derived from the bbox using the same calibrated baseline

# as size estimation). Requires config.PIXEL_TO_CM_BASELINE.

try:

minor_px = max(1, min(abs(x2 - x1), abs(y2 - y1)))

map_w = depth_map.shape[1] if depth_map.ndim == 2 else 1

dens = (getattr(config, "PIXEL_TO_CM_BASELINE", 0.35) *

(640.0 / map_w if map_w else 1.0))

minor_cm = minor_px * dens

depth_cm = min(depth_cm, round(0.5 * minor_cm, 2))

except Exception:

pass

depth_cm = round(max(depth_cm, 0.0), 2)

# Classify depth

depth_category = self._classify_depth(depth_cm)

return depth_cm, depth_category

def _classify_depth(self, depth_cm: float) -> str:

"""Classify depth into categories."""

if depth_cm < config.DEPTH_SHALLOW_MAX_CM:

return "Shallow"

elif depth_cm < config.DEPTH_MODERATE_MAX_CM:

return "Moderate"

else:

return "Deep"

def create_depth_visualization(self, depth_map: np.ndarray) -> np.ndarray:

"""Create a color-mapped visualization of the depth map."""

depth_normalized = depth_map.copy()

depth_min = depth_normalized.min()

depth_max = depth_normalized.max()

if depth_max > depth_min:

depth_normalized = (depth_normalized - depth_min) / (depth_max - depth_min)

depth_uint8 = (depth_normalized * 255).astype(np.uint8)

depth_colored = cv2.applyColorMap(depth_uint8, cv2.COLORMAP_JET)

return depth_colored

def estimate_depth_for_pothole(self, image, bbox):

"""Full pipeline: estimate depth map and extract pothole depth."""

depth_map = self.estimate_depth(image)

depth_cm, category = self.get_pothole_depth(depth_map, bbox)

return DepthResult(

depth_map=depth_map,

relative_depth=(depth_map - depth_map.min()) /

(depth_map.max() - depth_map.min() + 1e-8),

pothole_depth_cm=depth_cm,

depth_category=category,

)

# src/size_estimator.py

Pixel -> cm size estimation (camera model) + Small/Medium/Large

"""

Size Estimation Module

======================

Estimates pothole size (length, width, area) in real-world units

using pixel measurements and camera calibration parameters.

Classes:

- SizeEstimator: Estimates real-world pothole dimensions

Usage:

from src.size_estimator import SizeEstimator

estimator = SizeEstimator()

width_cm, height_cm, area_cm2 = estimator.estimate_size(bbox, image_width)

"""

import numpy as np

from typing import Tuple, Optional

from dataclasses import dataclass

import config

from .utils import setup_logger

logger = setup_logger(__name__)

@dataclass

class SizeResult:

"""Container for size estimation results."""

width_px: int = 0

height_px: int = 0

width_cm: float = 0.0

height_cm: float = 0.0

area_cm2: float = 0.0

size_category: str = ""

pixel_density_cm_per_px: float = 0.0

class SizeEstimator:

"""

Real-world size estimation for potholes.

Uses camera parameters and reference measurements to convert

pixel coordinates to physical dimensions (cm).

"""

def __init__(self,

focal_length: float = None,

camera_height_cm: float = None,

reference_width_cm: float = None):

"""

Initialize size estimator.

Args:

focal_length: Camera focal length in pixels

camera_height_cm: Camera height from road surface (cm)

reference_width_cm: Known reference object width (cm)

"""

self.focal_length = focal_length or config.FOCAL_LENGTH_PIXELS

self.camera_height_cm = camera_height_cm or config.CAMERA_HEIGHT_CM

self.reference_width_cm = reference_width_cm or config.REFERENCE_OBJECT_SIZE_CM

def estimate_size(self, bbox: Tuple[int, int, int, int],

image_width: int,

reference_bbox: Tuple[int, int, int, int] = None) -> SizeResult:

"""

Estimate real-world pothole size from bounding box.

Args:

bbox: (x1, y1, x2, y2) bounding box in pixels

image_width: Width of the source image in pixels

reference_bbox: Optional reference object bbox for calibration

Returns:

SizeResult with all size measurements

"""

x1, y1, x2, y2 = bbox

width_px = x2 - x1

height_px = y2 - y1

area_px = width_px * height_px

# Calculate pixel-to-cm conversion factor

pixel_density = self._calculate_pixel_density(

image_width, reference_bbox

)

width_cm = width_px * pixel_density

height_cm = height_px * pixel_density

area_cm2 = width_cm * height_cm

size_category = self._classify_size(area_cm2)

logger.info(

f"Size: {width_cm:.1f}x{height_cm:.1f}cm, "

f"Area: {area_cm2:.1f}cm2, Category: {size_category}"

)

return SizeResult(

width_px=width_px,

height_px=height_px,

width_cm=round(width_cm, 2),

height_cm=round(height_cm, 2),

area_cm2=round(area_cm2, 2),

size_category=size_category,

pixel_density_cm_per_px=round(pixel_density, 6),

)

def _calculate_pixel_density(self, image_width: int,

reference_bbox: Tuple[int, int, int, int] = None

) -> float:

"""

Calculate pixel-to-cm density.

Uses reference object if provided, otherwise estimates

based on camera parameters and geometry.

"""

if reference_bbox is not None:

ref_x1, ref_y1, ref_x2, ref_y2 = reference_bbox

ref_width_px = abs(ref_x2 - ref_x1)

if ref_width_px > 0:

return self.reference_width_cm / ref_width_px

# Fallback: calibrated baseline estimate.

# IMPORTANT: For accurate measurements, pass a reference_bbox with a

# known object width, or calibrate config.PIXEL_TO_CM_BASELINE for

# your specific camera and mounting height.

baseline_640 = config.PIXEL_TO_CM_BASELINE

# Same scene at higher resolution => each pixel covers LESS ground.

# scale_factor = W / 640, so cm-per-pixel = baseline / scale_factor.

scale_factor = image_width / 640.0 if image_width > 0 else 1.0

pixel_to_cm = baseline_640 / scale_factor

return pixel_to_cm

def _classify_size(self, area_cm2: float) -> str:

"""Classify pothole size category based on area."""

if area_cm2 < config.SIZE_SMALL_MAX_CM2:

return "Small"

elif area_cm2 < config.SIZE_MEDIUM_MAX_CM2:

return "Medium"

else:

return "Large"

def estimate_depth_from_size_ratio(self, bbox: Tuple[int, int, int, int],

depth_value: float) -> float:

"""

Rough depth estimate based on depth map value and size ratio.

Args:

bbox: Bounding box

depth_value: Depth map value at pothole center

Returns:

Estimated depth in cm

"""

# This is a supplementary method for cross-validation

x1, y1, x2, y2 = bbox

width_px = x2 - x1

height_px = y2 - y1

# Larger potholes tend to be deeper (empirical observation)

area_factor = min(width_px * height_px / 10000.0, 5.0)

depth_estimate = depth_value * 5.0 * area_factor  # Scale to cm

return round(depth_estimate, 2)

def batch_estimate(self, bboxes: list, image_width: int) -> list:

"""Estimate sizes for a batch of bounding boxes."""

results = []

for bbox in bboxes:

result = self.estimate_size(bbox, image_width)

results.append(result)

return results

# src/severity.py

Severity scoring (0-100) + matrix classification + priority levels

"""

Severity Classification Module

==============================

Combines size and depth measurements to classify pothole

severity and assign maintenance priority levels.

Classes:

- SeverityClassifier: Classifies pothole severity

- SeverityResult: Container for classification output

Usage:

from src.severity import SeverityClassifier

classifier = SeverityClassifier()

result = classifier.classify(size_result, depth_result)

print(result.severity, result.priority)

"""

from typing import Tuple, Optional

from dataclasses import dataclass

import config

from .utils import setup_logger

logger = setup_logger(__name__)

@dataclass

class SeverityResult:

"""Container for severity classification results."""

severity: str                # Critical, High, Medium, Low

size_category: str           # Small, Medium, Large

depth_category: str          # Shallow, Moderate, Deep

priority: int                # 1=Critical, 4=Low (lower = more urgent)

color_bgr: Tuple[int, int, int]  # BGR color for visualization

response_time_hours: int     # Recommended response deadline

recommendation: str          # Maintenance recommendation text

score: float = 0.0           # Numerical severity score (0-100)

class SeverityClassifier:

"""

Classify pothole severity based on size and depth.

Uses a rule-based system combined with configurable thresholds

from the config module.

"""

def __init__(self):

"""Initialize the severity classifier with config thresholds."""

self.size_thresholds = config.SIZE_CATEGORIES

self.depth_thresholds = config.DEPTH_CATEGORIES

self.severity_levels = config.SEVERITY_LEVELS

logger.info("SeverityClassifier initialized")

def classify(self, size_result=None, depth_cm: float = None,

depth_category: str = None, area_cm2: float = None,

size_category: str = None) -> SeverityResult:

"""

Classify pothole severity.

Can accept either SizeResult/DepthResult objects or raw values.

Args:

size_result: SizeResult object (optional)

depth_cm: Depth in centimeters

depth_category: Depth category string

area_cm2: Pothole area in cm²

size_category: Size category string

Returns:

SeverityResult with full classification

"""

# Extract size info

if size_result is not None:

area_cm2 = size_result.area_cm2

size_category = size_result.size_category

elif area_cm2 is None or size_category is None:

area_cm2 = 0

size_category = "Unknown"

# Extract depth info

if depth_cm is not None and depth_category is None:

depth_category = self._category_from_depth(depth_cm)

if depth_cm is None:

depth_cm = 0

if depth_category is None:

depth_category = "Shallow"

# Calculate severity score (0-100)

score = self._calculate_score(area_cm2, depth_cm, size_category, depth_category)

# Determine severity level

severity, priority, color, response_hours = self._determine_severity(score)

# Generate recommendation

recommendation = self._generate_recommendation(

severity, size_category, depth_category

)

logger.info(

f"Severity: {severity} (Priority {priority}), "

f"Score: {score:.1f}, Size: {size_category}, Depth: {depth_category}"

)

return SeverityResult(

severity=severity,

size_category=size_category,

depth_category=depth_category,

priority=priority,

color_bgr=color,

response_time_hours=response_hours,

recommendation=recommendation,

score=round(score, 2),

)

def _calculate_score(self, area_cm2: float, depth_cm: float,

size_category: str, depth_category: str) -> float:

"""

Calculate severity score (0-100).

Combines size and depth with weighted scoring.

"""

# Size score (0-50)

if size_category == "Small":

size_score = min(area_cm2 / config.SIZE_SMALL_MAX_CM2 * 15, 15)

elif size_category == "Medium":

ratio = (area_cm2 - config.SIZE_SMALL_MAX_CM2) / \

(config.SIZE_MEDIUM_MAX_CM2 - config.SIZE_SMALL_MAX_CM2)

size_score = 15 + ratio * 25  # 15-40

else:

ratio = min((area_cm2 - config.SIZE_MEDIUM_MAX_CM2) / 2000, 1.0)

size_score = 40 + ratio * 10  # 40-50

# Depth score (0-50)

if depth_category == "Shallow":

depth_score = min(depth_cm / config.DEPTH_SHALLOW_MAX_CM * 10, 10)

elif depth_category == "Moderate":

ratio = (depth_cm - config.DEPTH_SHALLOW_MAX_CM) / \

(config.DEPTH_MODERATE_MAX_CM - config.DEPTH_SHALLOW_MAX_CM)

depth_score = 10 + ratio * 30  # 10-40

else:

ratio = min((depth_cm - config.DEPTH_MODERATE_MAX_CM) / 5.0, 1.0)

depth_score = 40 + ratio * 10  # 40-50

return size_score + depth_score

def _determine_severity(self, score: float) -> Tuple[str, int, Tuple[int, int, int], int]:

"""Map severity score to severity level, priority, and color."""

if score >= 80:

return ("Critical", 1, (0, 0, 255), 24)

elif score >= 60:

return ("High", 2, (0, 165, 255), 168)

elif score >= 35:

return ("Medium", 3, (0, 255, 255), 720)

else:

return ("Low", 4, (0, 255, 0), 2160)

def _category_from_depth(self, depth_cm: float) -> str:

"""Determine depth category from raw depth value."""

if depth_cm < config.DEPTH_SHALLOW_MAX_CM:

return "Shallow"

elif depth_cm < config.DEPTH_MODERATE_MAX_CM:

return "Moderate"

else:

return "Deep"

def _generate_recommendation(self, severity: str,

size_category: str,

depth_category: str) -> str:

"""Generate a maintenance recommendation string."""

if severity == "Critical":

return (f"CRITICAL: Immediate repair required. "

f"Large ({size_category}) and/or deep ({depth_category}) pothole "

f"posing severe safety risk. Dispatch crew within 24h.")

elif severity == "High":

return (f"HIGH PRIORITY: Schedule repair within 1 week. "

f"Size: {size_category}, Depth: {depth_category}. "

f"Monitor for rapid deterioration.")

elif severity == "Medium":

return (f"MEDIUM PRIORITY: Plan repair within 1 month. "

f"Size: {size_category}, Depth: {depth_category}. "

f"Include in quarterly maintenance schedule.")

else:

return (f"LOW PRIORITY: Routine monitoring. "

f"Size: {size_category}, Depth: {depth_category}. "

f"Include in annual maintenance planning.")

def classify_matrix(self, size_category: str, depth_category: str) -> SeverityResult:

"""

Classify using a pre-defined severity matrix.

This provides an alternative rule-based approach

for quick severity assignment.

"""

matrix = {

("Small", "Shallow"): ("Low", 4, 2160),

("Small", "Moderate"): ("Medium", 3, 720),

("Small", "Deep"): ("High", 2, 168),

("Medium", "Shallow"): ("Medium", 3, 720),

("Medium", "Moderate"): ("High", 2, 168),

("Medium", "Deep"): ("Critical", 1, 24),

("Large", "Shallow"): ("High", 2, 168),

("Large", "Moderate"): ("Critical", 1, 24),

("Large", "Deep"): ("Critical", 1, 24),

}

if (size_category, depth_category) in matrix:

sev, pri, hours = matrix[(size_category, depth_category)]

else:

sev, pri, hours = ("Low", 4, 2160)

colors = {

"Critical": (0, 0, 255),

"High": (0, 165, 255),

"Medium": (0, 255, 255),

"Low": (0, 255, 0),

}

recommendation = self._generate_recommendation(sev, size_category, depth_category)

return SeverityResult(

severity=sev,

size_category=size_category,

depth_category=depth_category,

priority=pri,

color_bgr=colors[sev],

response_time_hours=hours,

recommendation=recommendation,

score=0.0,

)

# src/utils.py

Helpers: logging, image IO, drawing boxes, IoU, report generation

"""

Utility functions for the Road Pothole Detection System.

Provides helper functions for:

- File I/O and path management

- Image conversion and visualization

- Coordinate transformations

- Logging setup

- Data serialization

"""

import os

import cv2

import json

import logging

import numpy as np

from datetime import datetime

from typing import Tuple, List, Optional, Dict, Any

import config

# ============================================================

# Logging Setup

# ============================================================

def setup_logger(name: str, log_file: Optional[str] = None, level: int = logging.INFO) -> logging.Logger:

"""

Set up a logger with console and file handlers.

Args:

name: Logger name

log_file: Optional path to log file

level: Logging level

Returns:

Configured logger instance

"""

logger = logging.getLogger(name)

logger.setLevel(level)

# Avoid duplicate handlers

if logger.handlers:

return logger

formatter = logging.Formatter(

"%(asctime)s - %(name)s - %(levelname)s - %(message)s",

datefmt="%Y-%m-%d %H:%M:%S"

)

# Console handler

console_handler = logging.StreamHandler()

console_handler.setLevel(level)

console_handler.setFormatter(formatter)

logger.addHandler(console_handler)

# File handler

if log_file:

file_handler = logging.FileHandler(log_file)

file_handler.setLevel(level)

file_handler.setFormatter(formatter)

logger.addHandler(file_handler)

return logger

# ============================================================

# Image Utilities

# ============================================================

def load_image(image_path: str) -> Optional[np.ndarray]:

"""

Load an image from file path.

Args:

image_path: Path to image file

Returns:

Image as numpy array (BGR format) or None if loading fails

"""

if not os.path.exists(image_path):

raise FileNotFoundError(f"Image not found: {image_path}")

image = cv2.imread(image_path)

if image is None:

raise ValueError(f"Failed to load image: {image_path}")

return image

def load_image_from_bytes(image_bytes: bytes) -> Optional[np.ndarray]:

"""

Load an image from raw bytes.

Args:

image_bytes: Raw image bytes

Returns:

Image as numpy array (BGR format) or None

"""

nparr = np.frombuffer(image_bytes, np.uint8)

image = cv2.imdecode(nparr, cv2.IMREAD_COLOR)

return image

def save_image(image: np.ndarray, output_path: str) -> bool:

"""

Save an image to file.

Args:

image: Image as numpy array

output_path: Output file path

Returns:

True if saved successfully, False otherwise

"""

dir_path = os.path.dirname(output_path)

if dir_path:

os.makedirs(dir_path, exist_ok=True)

return cv2.imwrite(output_path, image)

def image_to_base64(image: np.ndarray) -> str:

"""Convert a numpy image array to base64 string."""

import base64

_, buffer = cv2.imencode(".jpg", image)

return base64.b64encode(buffer).decode("utf-8")

def base64_to_image(base64_str: str) -> np.ndarray:

"""Convert a base64 string to numpy image array."""

import base64

image_data = base64.b64decode(base64_str)

nparr = np.frombuffer(image_data, np.uint8)

return cv2.imdecode(nparr, cv2.IMREAD_COLOR)

def resize_image(image: np.ndarray, max_dim: int = 640) -> np.ndarray:

"""

Resize image while maintaining aspect ratio.

Args:

image: Input image

max_dim: Maximum dimension (height or width)

Returns:

Resized image

"""

h, w = image.shape[:2]

scale = min(max_dim / w, max_dim / h, 1.0)

new_w, new_h = int(w * scale), int(h * scale)

return cv2.resize(image, (new_w, new_h), interpolation=cv2.INTER_AREA)

# ============================================================

# Coordinate & Bounding Box Utilities

# ============================================================

def extract_bounding_boxes(results, image_shape: Tuple[int, int]) -> List[Dict[str, Any]]:

"""

Extract bounding boxes from YOLO detection results.

Args:

results: YOLO detection results

image_shape: (height, width) of the source image

Returns:

List of bounding box dictionaries

"""

boxes = []

img_h, img_w = image_shape[:2]

if results.boxes is not None:

for box in results.boxes:

xyxy = box.xyxy[0].cpu().numpy()

conf = float(box.conf.cpu().numpy()[0])

cls_id = int(box.cls.cpu().numpy()[0])

x1, y1, x2, y2 = xyxy

width_px = int(x2 - x1)

height_px = int(y2 - y1)

x1 = max(0, int(x1))

y1 = max(0, int(y1))

x2 = min(img_w - 1, int(x2))

y2 = min(img_h - 1, int(y2))

class_name = config.CLASS_NAMES.get(cls_id, f"class_{cls_id}")

boxes.append({

"bbox": [x1, y1, x2, y2],

"confidence": conf,

"class_id": cls_id,

"class_name": class_name,

"width_px": width_px,

"height_px": height_px,

"center": [(x1 + x2) // 2, (y1 + y2) // 2],

})

return boxes

def pixels_to_cm(pixel_width: float, pixel_height: float,

reference_width_px: float = None,

reference_width_cm: float = None,

distance_m: float = 2.0) -> Tuple[float, float]:

"""Convert pixel measurements to real-world centimeters."""

if reference_width_px and reference_width_cm and reference_width_px > 0:

pixel_to_cm = reference_width_cm / reference_width_px

width_cm = pixel_width * pixel_to_cm

height_cm = pixel_height * pixel_to_cm

else:

f = config.FOCAL_LENGTH_PIXELS

pixel_to_cm = (distance_m * 100) / f

width_cm = pixel_width * pixel_to_cm

height_cm = pixel_height * pixel_to_cm

return width_cm, height_cm

def draw_bounding_boxes(image: np.ndarray, detections: List[Dict[str, Any]],

thickness: int = 2) -> np.ndarray:

"""Draw bounding boxes with labels on image."""

img = image.copy()

for det in detections:

x1, y1, x2, y2 = det["bbox"]

conf = det["confidence"]

label = det.get("class_name", "Object")

severity = det.get("severity", "")

size_cat = det.get("size_category", "")

if severity == "Critical":

color = (0, 0, 255)

elif severity == "High":

color = (0, 165, 255)

elif severity == "Medium":

color = (0, 255, 255)

else:

color = (0, 255, 0)

cv2.rectangle(img, (x1, y1), (x2, y2), color, thickness)

label_text = f"{label}: {conf:.2f}"

if size_cat:

label_text += f" [{size_cat}]"

(w, h), _ = cv2.getTextSize(label_text, cv2.FONT_HERSHEY_SIMPLEX, 0.6, 1)

cv2.rectangle(img, (x1, y1 - 20), (x1 + w, y1), color, -1)

cv2.putText(img, label_text, (x1, y1 - 5),

cv2.FONT_HERSHEY_SIMPLEX, 0.6, (255, 255, 255), 1)

if severity:

sev_text = f"Severity: {severity}"

cv2.rectangle(img, (x1, y2), (x1 + 120, y2 + 20), color, -1)

cv2.putText(img, sev_text, (x1 + 5, y2 + 15),

cv2.FONT_HERSHEY_SIMPLEX, 0.5, (255, 255, 255), 1)

return img

# ============================================================

# Report Generation

# ============================================================

def generate_report_json(detections: List[Dict[str, Any]],

image_path: str = "",

gps_coords: Tuple[float, float] = None) -> Dict[str, Any]:

"""Generate a JSON-serializable report from detections."""

report = {

"timestamp": datetime.now().isoformat(),

"image_path": image_path,

"gps_coordinates": {"lat": gps_coords[0], "lng": gps_coords[1]} if gps_coords else None,

"total_potholes_detected": len(detections),

"potholes": detections,

"summary": {

"critical_count": sum(1 for d in detections if d.get("severity") == "Critical"),

"high_count": sum(1 for d in detections if d.get("severity") == "High"),

"medium_count": sum(1 for d in detections if d.get("severity") == "Medium"),

"low_count": sum(1 for d in detections if d.get("severity") == "Low"),

}

}

return report

def format_severity_report(report: Dict[str, Any]) -> str:

"""Format a report dictionary into a human-readable string."""

lines = [

"=" * 60,

"POTHOLE DETECTION & SEVERITY REPORT",

"=" * 60,

f"Timestamp: {report['timestamp']}",

f"Image: {report.get('image_path', 'N/A')}",

f"GPS: {report.get('gps_coordinates', 'N/A')}",

f"Total Potholes Detected: {report['total_potholes_detected']}",

"",

"SEVERITY SUMMARY:",

f"  [CRITICAL]: {report['summary']['critical_count']}",

f"  [HIGH]:      {report['summary']['high_count']}",

f"  [MEDIUM]:    {report['summary']['medium_count']}",

f"  [LOW]:       {report['summary']['low_count']}",

"",

"DETAILED FINDINGS:",

"-" * 60,

]

for i, p in enumerate(report["potholes"], 1):

lines.append(f"\nPothole #{i}:")

lines.append(f"  Location (px): bbox={p['bbox']}")

lines.append(f"  Confidence: {p['confidence']:.4f}")

lines.append(f"  Size: {p.get('size_cm2', 'N/A')} cm2 ({p.get('size_category', 'N/A')})")

lines.append(f"  Dimensions: {p.get('width_cm', 'N/A')}cm x {p.get('height_cm', 'N/A')}cm")

lines.append(f"  Depth Est.: {p.get('depth_cm', 'N/A')} cm ({p.get('depth_category', 'N/A')})")

lines.append(f"  Severity: {p.get('severity', 'Unknown')}")

lines.append(f"  Priority: {p.get('priority', 'N/A')}")

lines.append("\n" + "=" * 60)

return "\n".join(lines)

def save_json(data: Dict[str, Any], filepath: str) -> None:

"""Save dictionary as JSON file."""

dir_path = os.path.dirname(filepath)

if dir_path:

os.makedirs(dir_path, exist_ok=True)

with open(filepath, "w") as f:

json.dump(data, f, indent=2, default=str)

def load_json(filepath: str) -> Dict[str, Any]:

"""Load JSON file as dictionary."""

if not os.path.exists(filepath):

return {}

with open(filepath, "r") as f:

return json.load(f)

# ============================================================

# Performance Metrics

# ============================================================

def calculate_iou(box1: List[int], box2: List[int]) -> float:

"""Calculate Intersection over Union (IoU) between two bounding boxes."""

x1_i = max(box1[0], box2[0])

y1_i = max(box1[1], box2[1])

x2_i = min(box1[2], box2[2])

y2_i = min(box1[3], box2[3])

intersection_area = max(0, x2_i - x1_i) * max(0, y2_i - y1_i)

box1_area = (box1[2] - box1[0]) * (box1[3] - box1[1])

box2_area = (box2[2] - box2[0]) * (box2[3] - box2[1])

union_area = box1_area + box2_area - intersection_area

if union_area == 0:

return 0.0

return intersection_area / union_area

# ============================================================

# Validation Utilities

# ============================================================

def is_valid_image(filepath: str) -> bool:

"""Check if file is a valid image."""

try:

img = load_image(filepath)

return img is not None

except Exception:

return False

def get_file_size_mb(filepath: str) -> float:

"""Get file size in megabytes."""

if os.path.exists(filepath):

return os.path.getsize(filepath) / (1024 * 1024)

return 0.0

def allowed_file(filename: str) -> bool:

"""Check if file extension is allowed."""

return "." in filename and \

filename.rsplit(".", 1)[1].lower() in config.ALLOWED_EXTENSIONS

# api/app.py

Flask REST API: /api/analyze, /api/detect, /api/reports, /dashboard

"""

Flask API Application for Pothole Detection

==========================================

REST API endpoints for automated pothole detection and severity analysis.

Endpoints:

GET  /          - API info

GET  /dashboard - Web dashboard

POST /api/detect - Detect potholes in uploaded image

POST /api/analyze - Full analysis (detection + size + depth + severity)

GET  /api/health - Health check

GET  /api/reports - List reports

GET  /api/report/<id> - Get specific report

Usage:

python api/app.py

or

python main.py --web

"""

import os

import io

import json

import sys

import uuid

# Ensure the project root is on sys.path so that `python api/app.py` works.

# Running a script inside a subpackage only puts that subpackage's directory

# on sys.path by default, which would break `import config` and `import src`.

PROJECT_ROOT = os.path.dirname(os.path.dirname(os.path.abspath(__file__)))

if PROJECT_ROOT not in sys.path:

sys.path.insert(0, PROJECT_ROOT)

import cv2

import numpy as np

from datetime import datetime

from typing import Tuple, Optional

from glob import glob

from threading import Thread

from flask import Flask, request, jsonify, render_template, \

send_from_directory, Response, url_for

from flask_cors import CORS

from werkzeug.utils import secure_filename

import config

from src.detector import PotholeDetector, DetectionResult

from src.depth_estimator import DepthEstimator

from src.size_estimator import SizeEstimator

from src.severity import SeverityClassifier

from main import PotholeAnalysisPipeline

from src.utils import setup_logger, load_image_from_bytes, save_image, \

image_to_base64, save_json, allowed_file

logger = setup_logger("api", config.LOG_FILE)

app = Flask(

__name__,

template_folder=os.path.join(os.path.dirname(__file__), "..", "frontend", "templates"),

static_folder=os.path.join(os.path.dirname(__file__), "..", "frontend", "static"),

static_url_path="/static",

)

app.config["MAX_CONTENT_LENGTH"] = config.MAX_CONTENT_LENGTH_MB * 1024 * 1024

app.config["SECRET_KEY"] = config.SECRET_KEY

app.config["UPLOAD_FOLDER"] = config.UPLOADS_DIR

CORS(app)

# Ensure upload directory exists

os.makedirs(app.config["UPLOAD_FOLDER"], exist_ok=True)

# Lazy-load pipeline (loaded on first request to handle model downloads)

_pipeline = None

def get_pipeline() -> PotholeAnalysisPipeline:

"""Lazy-initialize the analysis pipeline."""

global _pipeline

if _pipeline is None:

logger.info("Initializing analysis pipeline...")

_pipeline = PotholeAnalysisPipeline()

logger.info("Pipeline ready")

return _pipeline

# ============================================================

# Routes

# ============================================================

@app.route("/")

def index():

"""API root endpoint."""

return jsonify({

"service": "Pothole Detection & Severity Analysis API",

"version": "1.0.0",

"endpoints": {

"/api/health": "GET - Health check",

"/api/detect": "POST - Upload image, get detection results",

"/api/analyze": "POST - Full analysis (detect + size + depth + severity)",

"/dashboard": "GET - Web dashboard",

"/api/reports": "GET - List all reports",

},

})

@app.route("/dashboard")

def dashboard():

"""Serve the web dashboard."""

return render_template("index.html")

@app.route("/api/health")

def health_check():

"""Health check endpoint."""

return jsonify({

"status": "healthy",

"timestamp": datetime.now().isoformat(),

"device": config.DEFAULT_DEVICE,

})

@app.route("/api/detect", methods=["POST"])

def detect_potholes():

"""

Detect potholes in an uploaded image.

Expects: multipart/form-data with 'image' file

Optional: 'gps_lat', 'gps_lng' as form fields

Returns:

JSON with detection results and annotated image (base64)

"""

if "image" not in request.files:

return jsonify({"error": "No image file provided"}), 400

file = request.files["image"]

if file.filename == "":

return jsonify({"error": "No file selected"}), 400

if not allowed_file(file.filename):

return jsonify({"error": "File type not allowed"}), 400

# Load image

image_bytes = file.read()

image = load_image_from_bytes(image_bytes)

if image is None:

return jsonify({"error": "Failed to load image"}), 400

# Get GPS coords if provided

gps_lat = request.form.get("gps_lat", type=float)

gps_lng = request.form.get("gps_lng", type=float)

gps = (gps_lat, gps_lng) if gps_lat and gps_lng else None

try:

pipeline = get_pipeline()

result = pipeline.analyze_image(image, gps_coords=gps, visualize=True)

# Save annotated image

filename = secure_filename(file.filename)

unique_id = str(uuid.uuid4())[:8]

annotated_filename = f"{os.path.splitext(filename)[0]}_{unique_id}_annotated.jpg"

annotated_path = os.path.join(app.config["UPLOAD_FOLDER"], annotated_filename)

save_image(result["annotated_image"], annotated_path)

# Save report

report_filename = f"{os.path.splitext(filename)[0]}_{unique_id}_report.json"

report_path = os.path.join(app.config["UPLOAD_FOLDER"], report_filename)

save_json(result["report"], report_path)

# Add paths to response

result["report"]["annotated_image_path"] = annotated_path

result["report"]["report_path"] = report_path

result["report"]["report_id"] = unique_id

return jsonify(result["report"])

except Exception as e:

logger.error(f"Detection failed: {e}")

return jsonify({"error": str(e)}), 500

@app.route("/api/analyze", methods=["POST"])

def analyze_pothole():

"""

Full analysis: detection + size + depth + severity.

Expects: multipart/form-data with 'image' file

Optional: 'gps_lat', 'gps_lng' as form fields

Returns:

JSON with complete analysis report and base64 annotated image

"""

if "image" not in request.files:

return jsonify({"error": "No image file provided"}), 400

file = request.files["image"]

if file.filename == "":

return jsonify({"error": "No file selected"}), 400

image_bytes = file.read()

image = load_image_from_bytes(image_bytes)

if image is None:

return jsonify({"error": "Failed to load image"}), 400

gps_lat = request.form.get("gps_lat", type=float)

gps_lng = request.form.get("gps_lng", type=float)

gps = (gps_lat, gps_lng) if gps_lat and gps_lng else None

try:

pipeline = get_pipeline()

result = pipeline.analyze_image(image, gps_coords=gps, visualize=True)

# Convert annotated image to base64

annotated_b64 = image_to_base64(result["annotated_image"])

response = {

"status": "success",

"timestamp": datetime.now().isoformat(),

"image_info": {

"height": image.shape[0],

"width": image.shape[1],

},

"detections": result["detections"],

"annotated_image_base64": annotated_b64,

"report": result["report"],

"processing_time_seconds": result["processing_time_seconds"],

}

return jsonify(response)

except Exception as e:

logger.error(f"Analysis failed: {e}")

return jsonify({"error": str(e), "status": "error"}), 500

@app.route("/api/reports", methods=["GET"])

def list_reports():

"""List all saved reports."""

upload_dir = app.config["UPLOAD_FOLDER"]

reports = []

for f in glob(os.path.join(upload_dir, "*_report.json")):

try:

with open(f, "r") as fh:

report = json.load(fh)

report["report_file"] = os.path.basename(f)

report["annotated_image_file"] = os.path.basename(

f).replace("_report.json", "_annotated.jpg")

reports.append(report)

except Exception as e:

logger.error(f"Failed to read report {f}: {e}")

return jsonify({"reports": reports, "count": len(reports)})

@app.route("/api/report/<report_id>", methods=["GET"])

def get_report(report_id):

"""Get a specific report by ID."""

upload_dir = app.config["UPLOAD_FOLDER"]

for f in glob(os.path.join(upload_dir, f"*_{report_id}_report.json")):

with open(f, "r") as fh:

report = json.load(fh)

return jsonify(report)

return jsonify({"error": "Report not found"}), 404

@app.route("/api/visualize", methods=["POST"])

def visualize_detections_endpoint():

"""Endpoint to just get the annotated image without full analysis."""

if "image" not in request.files:

return jsonify({"error": "No image file provided"}), 400

file = request.files["image"]

image_bytes = file.read()

image = load_image_from_bytes(image_bytes)

if image is None:

return jsonify({"error": "Failed to load image"}), 400

try:

pipeline = get_pipeline()

result = pipeline.analyze_image(image, gps_coords=None, visualize=True)

annotated_b64 = image_to_base64(result["annotated_image"])

return jsonify({

"status": "success",

"annotated_image_base64": annotated_b64,

"detections": result["detections"],

})

except Exception as e:

logger.error(f"Visualization failed: {e}")

return jsonify({"error": str(e)}), 500

@app.errorhandler(413)

def request_entity_too_large(error):

return jsonify({"error": "File too large (max 50MB)"}), 413

@app.errorhandler(404)

def not_found(error):

return jsonify({"error": "Endpoint not found"}), 404

@app.errorhandler(500)

def internal_error(error):

return jsonify({"error": "Internal server error"}), 500

if __name__ == "__main__":

app.run(

host=config.API_HOST,

port=config.API_PORT,

debug=config.API_DEBUG,

)

# train.py

YOLOv8 training script (dataset config, train, validate, export)

"""

YOLOv8 Training Script for Pothole Detection

=============================================

Trains a custom YOLOv8 model on pothole datasets.

Usage:

# 1. Create a dataset config template

python train.py --create-config

# 2. Prepare dataset in YOLO format:

#    dataset/

#      images/train/*.jpg

#      images/val/*.jpg

#      labels/train/*.txt

#      labels/val/*.txt

#      dataset.yaml

# 3. Train:

python train.py --data dataset.yaml --epochs 100 --model yolov8s.pt

# 4. Validate:

python train.py --validate runs/train/pothole_model/weights/best.pt

# 5. Export to ONNX:

python train.py --export runs/train/pothole_model/weights/best.pt

"""

import os

import sys

import argparse

import shutil

import yaml

try:

from ultralytics import YOLO

ULTRALYTICS_AVAILABLE = True

except ImportError:

ULTRALYTICS_AVAILABLE = False

import config

from src.utils import setup_logger

logger = setup_logger("train", config.LOG_FILE)

# Default dataset configuration template

DATASET_TEMPLATE = {

"path": "datasets/pothole_data",

"train": "images/train",

"val": "images/val",

"test": "images/test",

"names": {0: "Pothole"},

}

def create_dataset_yaml(output_path: str, dataset_root: str = None,

class_names: dict = None) -> str:

"""

Create a YOLO dataset configuration YAML file.

Args:

output_path: Path where to write the YAML

dataset_root: Root directory of the dataset

class_names: Dictionary of {class_id: class_name}

Returns:

Path to created YAML file

"""

cfg = DATASET_TEMPLATE.copy()

if dataset_root:

cfg["path"] = dataset_root

if class_names:

cfg["names"] = class_names

dir_path = os.path.dirname(output_path)

if dir_path:

os.makedirs(dir_path, exist_ok=True)

with open(output_path, "w") as f:

yaml.dump(cfg, f, default_flow_style=False)

logger.info(f"Dataset config created: {output_path}")

return output_path

def train_model(data_yaml, model_name="yolov8s.pt", epochs=100,

batch_size=16, img_size=640, device=None,

project="runs/train", name="pothole_model",

resume=False, patience=20):

"""

Train a YOLOv8 model for pothole detection.

Args:

data_yaml: Path to dataset configuration YAML

model_name: Base model to fine-tune (yolov8n/s/m/l/x.pt)

epochs: Number of training epochs

batch_size: Batch size

img_size: Input image size

device: Device ('cuda', 'cpu', or None for auto)

project: Output project directory

name: Run name

resume: Resume from last checkpoint

patience: Early stopping patience

Returns:

Training results

"""

if not ULTRALYTICS_AVAILABLE:

raise ImportError(

"Ultralytics is required for training. "

"Install with: pip install ultralytics"

)

if not os.path.exists(data_yaml):

raise FileNotFoundError(f"Dataset config not found: {data_yaml}")

if device is None:

device = config.DEFAULT_DEVICE

logger.info("=" * 60)

logger.info("STARTING YOLOv8 TRAINING")

logger.info("=" * 60)

logger.info(f"  Base model:   {model_name}")

logger.info(f"  Dataset:      {data_yaml}")

logger.info(f"  Epochs:       {epochs}")

logger.info(f"  Batch size:   {batch_size}")

logger.info(f"  Image size:   {img_size}")

logger.info(f"  Device:       {device}")

logger.info(f"  Output:       {project}/{name}")

model = YOLO(model_name)

results = model.train(

data=data_yaml,

epochs=epochs,

batch=batch_size,

imgsz=img_size,

device=device,

project=project,

name=name,

resume=resume,

patience=patience,

mosaic=1.0,

mixup=0.1,

hsv_h=0.015,

hsv_s=0.7,

hsv_v=0.4,

degrees=15.0,

translate=0.1,

scale=0.5,

fliplr=0.5,

optimizer="AdamW",

lr0=0.001,

lrf=0.01,

weight_decay=0.0005,

warmup_epochs=3.0,

verbose=True,

)

best_model = os.path.join(project, name, "weights", "best.pt")

if os.path.exists(best_model):

dest = config.YOLO_MODEL_PATH

os.makedirs(os.path.dirname(dest), exist_ok=True)

shutil.copy2(best_model, dest)

logger.info(f"Best model copied to: {dest}")

logger.info("Training complete!")

return results

def validate_model(model_path, data_yaml, device=None):

"""Validate a trained model on the validation set."""

if device is None:

device = config.DEFAULT_DEVICE

logger.info(f"Validating model: {model_path}")

model = YOLO(model_path)

metrics = model.val(data=data_yaml, device=device)

logger.info(f"  mAP@0.5:      {metrics.box.map50:.4f}")

logger.info(f"  mAP@0.5:0.95: {metrics.box.map:.4f}")

logger.info(f"  Precision:    {metrics.box.mp:.4f}")

logger.info(f"  Recall:       {metrics.box.mr:.4f}")

return metrics

def export_model(model_path, export_format="onnx"):

"""

Export a trained model to a deployment format.

Args:

model_path: Path to trained .pt model

export_format: onnx, tflite, torchscript, coreml, etc.

"""

logger.info(f"Exporting {model_path} to {export_format}...")

model = YOLO(model_path)

model.export(format=export_format)

logger.info(f"Export complete: {export_format}")

def main():

"""Command-line entry point for training."""

parser = argparse.ArgumentParser(

description="Train YOLOv8 model for pothole detection"

)

parser.add_argument("--data", type=str, default="dataset.yaml",

help="Path to dataset YAML config")

parser.add_argument("--model", type=str, default="yolov8s.pt",

help="Base model (yolov8n/s/m/l/x.pt)")

parser.add_argument("--epochs", type=int, default=100,

help="Number of training epochs")

parser.add_argument("--batch", type=int, default=16,

help="Batch size")

parser.add_argument("--imgsz", type=int, default=640,

help="Input image size")

parser.add_argument("--device", type=str, default=None,

help="Device: cuda, cpu, or 0,1 for multi-GPU")

parser.add_argument("--project", type=str, default="runs/train",

help="Output project directory")

parser.add_argument("--name", type=str, default="pothole_model",

help="Run name")

parser.add_argument("--resume", action="store_true",

help="Resume from last checkpoint")

parser.add_argument("--patience", type=int, default=20,

help="Early stopping patience")

parser.add_argument("--validate", type=str, default=None,

help="Validate a model at this path (skip training)")

parser.add_argument("--export", type=str, default=None,

help="Export model to format (onnx, tflite, etc.)")

parser.add_argument("--create-config", action="store_true",

help="Create a template dataset.yaml file")

args = parser.parse_args()

if args.create_config:

path = create_dataset_yaml("dataset.yaml")

print(f"\nDataset config template created: {path}")

print("Edit it to point to your dataset, then run training.")

return

if args.export:

export_model(args.export, "onnx")

return

if args.validate:

validate_model(args.validate, args.data, args.device)

return

train_model(

data_yaml=args.data,

model_name=args.model,

epochs=args.epochs,

batch_size=args.batch,

img_size=args.imgsz,

device=args.device,

project=args.project,

name=args.name,

resume=args.resume,

patience=args.patience,

)

if __name__ == "__main__":

main()

# tests/test_detector.py

Unit tests - 25 tests covering config, preprocessing, size, severity

"""

Unit Tests for Road Pothole Detection System

=============================================

Run with: pytest tests/ -v

Tests cover:

- Configuration validation

- Preprocessing functions

- Size estimation

- Severity classification

- Utility functions (bbox extraction, IoU, etc.)

"""

import os

import sys

import json

import numpy as np

import cv2

import pytest

# Add project root to path

sys.path.insert(0, os.path.dirname(os.path.dirname(os.path.abspath(__file__))))

import config

from src.utils import calculate_iou, generate_report_json

from src.size_estimator import SizeEstimator

from src.severity import SeverityClassifier

from src.preprocessing import enhance_contrast, reduce_noise, normalize_image

class TestConfig:

"""Test configuration constants."""

def test_config_values_exist(self):

assert hasattr(config, "CONFIDENCE_THRESHOLD")

assert hasattr(config, "IOU_THRESHOLD")

assert hasattr(config, "CLASS_NAMES")

assert hasattr(config, "SIZE_SMALL_MAX_CM2")

assert hasattr(config, "SIZE_MEDIUM_MAX_CM2")

assert hasattr(config, "DEPTH_SHALLOW_MAX_CM")

assert hasattr(config, "DEPTH_MODERATE_MAX_CM")

def test_confidence_threshold_range(self):

assert 0.0 < config.CONFIDENCE_THRESHOLD < 1.0

def test_size_thresholds_ordering(self):

assert config.SIZE_SMALL_MAX_CM2 < config.SIZE_MEDIUM_MAX_CM2

def test_depth_thresholds_ordering(self):

assert config.DEPTH_SHALLOW_MAX_CM < config.DEPTH_MODERATE_MAX_CM

def test_severity_levels_exist(self):

for level in ["Critical", "High", "Medium", "Low"]:

assert level in config.SEVERITY_LEVELS

class TestPreprocessing:

"""Test image preprocessing functions."""

@pytest.fixture

def sample_image(self):

"""Create a sample image for testing."""

img = np.random.randint(0, 255, (100, 100, 3), dtype=np.uint8)

return img

def test_enhance_contrast(self, sample_image):

enhanced = enhance_contrast(sample_image)

assert enhanced.shape == sample_image.shape

assert enhanced.dtype == sample_image.dtype

def test_reduce_noise(self, sample_image):

denoised = reduce_noise(sample_image)

assert denoised.shape == sample_image.shape

assert denoised.dtype == sample_image.dtype

def test_normalize_image(self, sample_image):

normalized = normalize_image(sample_image)

assert normalized.shape == sample_image.shape

assert normalized.dtype == np.float32

def test_preprocess_preserves_shape(self, sample_image):

from src.preprocessing import preprocess_image

result = preprocess_image(sample_image, normalize=False)

assert result.shape == sample_image.shape

class TestSizeEstimator:

"""Test size estimation logic."""

@pytest.fixture

def estimator(self):

return SizeEstimator()

def test_size_classification_small(self, estimator):

result = estimator.estimate_size((0, 0, 20, 20), 640)

assert result.size_category in ["Small", "Unknown"]

def test_size_classification_medium(self, estimator):

# Create a bbox that should produce medium size area

result = estimator.estimate_size((0, 0, 200, 200), 640)

assert result.size_category in ["Small", "Medium", "Large"]

def test_size_calculation_positive(self, estimator):

result = estimator.estimate_size((0, 0, 100, 100), 640)

assert result.width_cm > 0

assert result.height_cm > 0

assert result.area_cm2 > 0

def test_size_result_attributes(self, estimator):

result = estimator.estimate_size((10, 10, 110, 110), 640)

assert hasattr(result, "width_cm")

assert hasattr(result, "height_cm")

assert hasattr(result, "area_cm2")

assert hasattr(result, "size_category")

assert hasattr(result, "pixel_density_cm_per_px")

class TestSeverityClassifier:

"""Test severity classification logic."""

@pytest.fixture

def classifier(self):

return SeverityClassifier()

def test_low_severity_small_shallow(self, classifier):

result = classifier.classify(

area_cm2=100, size_category="Small",

depth_cm=1.0, depth_category="Shallow"

)

assert result.severity == "Low"

assert result.priority == 4

def test_critical_severity_large_deep(self, classifier):

result = classifier.classify(

area_cm2=5000, size_category="Large",

depth_cm=8.0, depth_category="Deep"

)

assert result.severity == "Critical"

assert result.priority == 1

def test_medium_severity_medium_moderate(self, classifier):

result = classifier.classify(

area_cm2=800, size_category="Medium",

depth_cm=3.5, depth_category="Moderate"

)

assert result.severity in ["Medium", "High"]

def test_severity_result_has_all_fields(self, classifier):

result = classifier.classify(

area_cm2=500, size_category="Medium",

depth_cm=2.5, depth_category="Moderate"

)

assert hasattr(result, "severity")

assert hasattr(result, "priority")

assert hasattr(result, "color_bgr")

assert hasattr(result, "response_time_hours")

assert hasattr(result, "recommendation")

def test_matrix_classification(self, classifier):

result = classifier.classify_matrix("Large", "Deep")

assert result.severity == "Critical"

def test_recommendation_generated(self, classifier):

result = classifier.classify(

area_cm2=5000, size_category="Large",

depth_cm=10.0, depth_category="Deep"

)

assert len(result.recommendation) > 0

assert "Immediate" in result.recommendation or "repair" in result.recommendation.lower()

class TestUtils:

"""Test utility functions."""

def test_calculate_iou_perfect_overlap(self):

box1 = [0, 0, 100, 100]

box2 = [0, 0, 100, 100]

assert calculate_iou(box1, box2) == 1.0

def test_calculate_iou_no_overlap(self):

box1 = [0, 0, 10, 10]

box2 = [100, 100, 200, 200]

assert calculate_iou(box1, box2) == 0.0

def test_calculate_iou_partial_overlap(self):

box1 = [0, 0, 100, 100]

box2 = [50, 50, 150, 150]

iou = calculate_iou(box1, box2)

assert 0.0 < iou < 1.0

def test_generate_report_structure(self):

detections = [

{"bbox": [10, 20, 50, 60], "confidence": 0.95,

"severity": "High", "size_cm2": 800, "size_category": "Medium"},

{"bbox": [100, 200, 150, 250], "confidence": 0.85,

"severity": "Low", "size_cm2": 200, "size_category": "Small"},

]

report = generate_report_json(detections, "test.jpg", (37.7749, -122.4194))

assert "timestamp" in report

assert "gps_coordinates" in report

assert "total_potholes_detected" in report

assert report["total_potholes_detected"] == 2

assert report["summary"]["high_count"] == 1

assert report["summary"]["low_count"] == 1

class TestDetectionResult:

"""Test DetectionResult dataclass."""

def test_detection_to_dict(self):

import sys

sys.path.insert(0, os.path.dirname(os.path.dirname(__file__)))

from src.detector import DetectionResult

det = DetectionResult(

bbox=[10, 20, 100, 200],

confidence=0.95,

class_id=0,

class_name="Pothole",

width_px=90,

height_px=180,

center=(55, 110),

size_cm2=750.0,

size_category="Medium",

width_cm=15.5,

height_cm=31.0,

depth_cm=3.5,

depth_category="Moderate",

severity="High",

priority=2,

response_time_hours=168,

)

d = det.to_dict()

assert d["class_name"] == "Pothole"

assert d["confidence"] == 0.95

assert d["severity"] == "High"

assert d["priority"] == 2

class TestIntegration:

"""Integration test for the full pipeline (requires torch)."""

@pytest.fixture

def real_image(self):

"""Load a sample road image if available."""

sample_paths = [

os.path.join(config.SAMPLES_DIR, "road1.jpg"),

os.path.join(config.SAMPLES_DIR, "road2.jpg"),

os.path.join(config.SAMPLES_DIR, "sample.jpg"),

]

for path in sample_paths:

if os.path.exists(path):

return cv2.imread(path)

# Create a dummy image if no sample exists

return np.random.randint(0, 255, (480, 640, 3), dtype=np.uint8)

@pytest.mark.skipif(not os.path.exists(config.YOLO_MODEL_PATH),

reason="YOLO model not available")

def test_full_pipeline(self, real_image):

"""Test the complete detection pipeline."""

from main import PotholeAnalysisPipeline

pipeline = PotholeAnalysisPipeline()

result = pipeline.analyze_image(real_image)

assert "detections" in result

assert "report" in result

assert "processing_time_seconds" in result

assert "annotated_image" in result

if __name__ == "__main__":

pytest.main([__file__, "-v", "--tb=short"])